ATAR Chemistry – Unit 1 & 2

Task 13   :   Research Assignment  – Quiz   SOLUTIONS

Chromatography

Part 1: Multiple-choice (answer by neatly writing your response in the column to the right)  (4 marks)

| - In Thin Layer Chromatography what does the symbol “RF” stand for? |  |
| --- | --- |
| - Chromatography factor | - B |
| - Retardation factor |  |
| - Retardation frequency |  |
| - Chromatography frequency |  |
| - High-performance Liquid chromatography is used to separate which of the following? |  |
| - Small organic molecules | - C |
| - Small covalent molecules |  |
| - Large organic molecules |  |
| - Large ionic molecules |  |
| - In High-performance Liquid chromatography the mobile phase is… |  |
| - Plasma | - C |
| - Gas |  |
| - Liquid |  |
| - Solid |  |
| - What is the stationary phase in Gas chromatography? |  |
| - A long, thin column. | - A |
| - Absorbent paper |  |
| - A short column |  |
| - Fine powder on glass or plastic |  |

- Part 2: Short Answer Questions:

- In thin film chromatography components of the mixture being analysed are separated.  ( 2 marks)

- What 2 processes allow the components to be separated?

    - Difference in solubility

    - Difference in adsorption to the adsorbent

- Draw a sketch of a Thin film chromatography set-up and label it. On your diagram

- include the terms  solvent, stationary phase, mobile phase, sample, analyte, absorbent material (adsorbent)

- (hint more than one word may be used for each part.)  (4 marks – 1 for correct diagram, ½ mark per label)

- Rf is characteristic of the material being analysed and can have a value between 0 and 1.     (2 marks)

- What does it mean when:

- Rf = 1:   No adsorption

- Rf = 0: No migration

4

a	i	X travelled fastest* because it had the smallest retention time*.

ii	Y is present in the highest concentration* because it has the largest peak area*.

(4 marks)

b	i	A calibration curve is necessary to relate the concentration of a substance to the area of the peak*.

ii	A solution of X of precise concentration is made up (1)*. This solution is used to prepare a set of standard diluted solutions of various concentrations* (1). Place these diluted solutions through the HPLC under the same conditions as the analysis of the pond water (1)*. Plot the area under the peak against the concentration of each standard* (1).                 (4 marks)

c	X and Z peaks at 8 minutes and 15 minutes respectively*.

X peak area is 10* and Z peak area is 20*.

(3 marks)

- Describe an application of gas Chromatography                                                ( 2marks )

Tests small molecular weight heat stable organic compounds such as ethanol   (1)

Applications, drug screening, oils in creams, toxicology, air borne pollutants, metabolic diseases  (1)

| - Gas Chromatography | - Long thin column | - Inert gas, eg N2, CO2 | - Small heat stable organic compounds such as ethanol |
| --- | --- | --- | --- |

Section B:   Water               Notes ____  /6

Part 1: Multiple-choice (answer by neatly writing your response in the column to the right)  [4 marks]

| - Why is chlorine added to drinking water? |  |
| --- | --- |
| - To remove heavy metals | - C |
| - To remove oxygen |  |
| - To kill various pathogens (bacteria, viruses etc) |  |
| - To make it slightly blue in colour. |  |
| - Which of the following elements is attributed to causing soil acidification? |  |
| - Nitrogen | - A |
| - Carbon |  |
| - Chlorine |  |
| - Fluorine |  |

- Part 2: Short Answer Questions:

- Describe, using chemical equations as appropriate, how increased levels of carbon dioxide in the atmosphere can cause acidification of oceans. [3]

- Increased concentrations of CO2 in the atmosphere increase concentrations of CO2 in water. [1]

- When CO2 dissolves in water it creates carbonic acid (according to the reaction) [1]

- The presence of carbonic acid decreases the pH

- For any source of drinking water in this state:   [Total 5]

  - Describe how the water is collected. [1]

- Must identify source with appropriate description, could include reservoirs, desalination, bores

  - Describe two steps take to make it potable/ safe to drink. [2]

- Two steps should be listed with a reason for the step. Could include

- Eg. Chlorination to kill bacteria etc

- Filtering to remove impurities

- Reverse osmosis   (desalination)

  - Any impacts on the environment. [2]

- Could include:

- Using groundwater can reduce water table, potential reduce water for plants and trees

- Desalination increases salinity of surrounding seawater affecting sea life

- Heavy Metal Contamination  [Total 5]

- a) Why is it necessary to monitor heavy metals?    [1]

- Toxic to humans, plants, animals

- b) Name 2 ways heavy metal contaminants could get into our water supply? ( 2 marks)

- Waste water from industrial processes

- Eg. As, Pb from gold mining

- Incorrect disposal of Pb Acid batteries etc

- c) Choose an industry and describe how they prevent/reduce water contamination? ( 3 marks)

- Names an industry and describes  2 processes. Could include

- eg. Redox processes to remove heavy metals

- Precipitation and ion exchange

- Discuss the causes, effects and remedies of Soil Acidification. [4 marks] – any 4 marks as long as all 3 points addressed

- Cause :  Increase in concentration of hydrogen ions

- Due to inefficient use of nitrogen, use of ammonium based fertilisers, removal of plants (any 1)

- Effects: Increase in solubility of Aluminium    (1), retards root growth and hence restricts access of water to the plant and restricts growth.(1)

- Remedies: Using lime, the CaCO3 and MgCO3 will reduce acidity by reacting with H+ (1)

- Management nitrogen fertiliser to reduce nitrate leaching

- Useful website: http://soilquality.org.au/factsheets/soil-acidity

- Extra info

- Heavy Metal: A metal with a relative density of 5.0 or higher.

- Heavy metals, such as mercury and lead, can cause various health problems.

- Eutrophication: The abundant growth of aquatic plants due to nutrient-enriched conditions, in particular, nitrate and phosphate enriched conditions.

- The aquatic plants that grow abundantly in eutrophication eventually use up all of the available nutrients that they require and die.

- The plants decompose, and in doing so, use up all dissolved oxygen.

- After using all oxygen, they decay anaerobically, resulting in chemicals that kill all remaining life.

- The decay causes sediment at the bottom of the water body.

- The mains sources of nutrients that cause eutrophication are:

  - Sewerage.

  - Fertiliser.

- Nitrate and phosphate are monitored in waterways vulnerable to eutrophication.

- The nitrogen-phosphorus ratio of waterways is often monitored, with the EPA recommending a ratio of less that 10:1.